# Assignment No. 3 — Dominant Eigenvalue and Eigenvector using Power Iteration (RV32IMF)

Course: Computer Architecture (ELL7282)

Level: 1st  Year M.Tech.

Target ISA: RV32IMF (RISC-V 32-bit Integer + Multiply/Divide + Float)

Expected Effort: <=15  hours

## 1. Learning Objectives

By completing this assignment, you will learn to:
- Implement a numerical linear-algebra algorithm in RISC-V assembly.
- Use RV32F floating-point operations, nested loops, and memory addressing.
- Apply the Power Iteration algorithm to compute the dominant eigenvalue and eigenvector.
- Evaluate algorithmic performance using instruction count, loop optimization, and register allocation.

## 2. Problem Statement

Write a RISC-V assembly program that estimates the dominant eigenvalue (λmax) and corresponding eigenvector of a real symmetric 6×6 matrix using the Power Iteration method (A·v = λv).

### Algorithm (Power Iteration)

Given A ∈ ℝ⁶ˣ⁶ and initial vector v(0):
Repeat for k = 1…Niter (max 10):
1. w = A × v(k−1)
2. λ(k) = (v(k−1))ᵀ w / (v(k−1))ᵀ v(k−1)
3. Normalize v(k) = w / ||w||
4. Stop if |λ(k) − λ(k−1)| < ε

### Constraints

- Use single-precision floats (.float, flw, fsw, fadd.s, fmul.s, fdiv.s, fsqrt.s).
- Matrix A is symmetric, stored row-major.
- Initial vector v_init = [1,1,1,1,1,1].
- Convergence tolerance ε = 1e−3, max 10 iterations.
- Output to memory: lambda_result (dominant eigenvalue), v_result[6] (normalized eigenvector).

## 3. Program Specification

Inputs: 6×6 matrix A, initial vector v_init, ε, Niter
Outputs: Float lambda_result, array v_result[6]
Subroutines: matvec_mul, dot_product, normalize_vector, power_iteration
Registers: Integers x5–x15 for indices; Floats f0–f15 for arithmetic.

## 4. Template Data Section

.data
N:          .word   6
epsilon:    .float  0.001
max_iter:   .word   10

A:  .float  6.0, 2.0, 0.0, 0.0, 0.0, 0.0,
            2.0, 5.0, 1.0, 0.0, 0.0, 0.0,
            0.0, 1.0, 4.0, 1.0, 0.0, 0.0,
            0.0, 0.0, 1.0, 3.0, 1.0, 0.0,
            0.0, 0.0, 0.0, 1.0, 2.0, 1.0,
            0.0, 0.0, 0.0, 0.0, 1.0, 1.0

v_init:     .float  1.0, 1.0, 1.0, 1.0, 1.0, 1.0
v_result:   .space  24
lambda_result: .float 0.0

## 5. Functional Requirements

Functions:
matvec_mul – Compute w = A×v (two nested loops).
dot_product – Compute vᵀ·w.
normalize_vector – Divide each component by ||w|| (fsqrt.s).
power_iteration – Perform iterative eigenvalue estimation and convergence check.
main – Initialize data, call subroutines, store λ and v.

## 6. Deliverables

1. Assembly file: power_iter_rv32imf.S
2. Execution evidence: screenshot or console dump showing λ and v_result.
3. Performance report (2 pages): instruction count, register allocation, loop optimization, precision analysis.
4. (Bonus +10 marks): use fmadd.s optimization.

## 7. Evaluation Rubric (100 Marks)

| Criterion | Marks | Description |
| --- | --- | --- |
| Correct eigenvalue/eigenvector computation | 25 | Produces correct λ, v within tolerance |
| RV32F floating-point ISA usage | 15 | Proper use of flw, fsw, fadd.s, fmul.s, fdiv.s |
| Modular subroutine design | 10 | JAL, stack usage, conventions |
| Matrix–vector and dot-product loops | 20 | Accurate addressing and iteration |
| Convergence logic | 10 | Proper float comparison and exit |
| Documentation | 10 | Readable, commented code |
| Performance report | 10 | Loop efficiency, instruction analysis |
| Bonus | +10 | fmadd.s or optimization |

## 8. Tools

Use either RARS or Venus (VS Code) simulators supporting RV32IMF.

## 9. Expected Output Example

Dominant Eigenvalue (λ): 7.8013
Eigenvector (v): [0.56, 0.49, 0.36, 0.25, 0.13, 0.07]
Iterations: 8
Converged: TRUE

## 10. Academic Integrity

All submissions must be individually authored. AI-generated or copied code will receive zero credit. Comments are mandatory for full marks.

# Appendix A — Installing and Using RARS

RARS (RISC-V Assembler and Runtime Simulator) is a Java-based IDE for writing and running RISC-V programs.

Installation:
1. Download from https://github.com/TheThirdOne/rars/releases
2. Run with:
   java -jar rars1_6.jar
3. Enable floating-point: Settings → Floating Point → Enabled (RV32F)

Usage:
- Open, assemble, run.
- Registers pane shows integer and float registers.
- Use 'li a7,2' + ecall to print float results.

# Appendix B — Using Venus with VS Code

Venus is a JavaScript-based RISC-V simulator that integrates with VS Code.

Setup:
1. Install VS Code → Extensions → 'Venus RISC-V Simulator'
2. Open your .S file → Click ▶ Run in Venus.
3. View output in the terminal window.

CLI Alternative:
   npm install -g venus
   venus-run your_file.S

Venus supports most RV32IMF instructions and is ideal for quick testing.

# Appendix C — Test Matrix and Expected Output

Matrix for Evaluation (A, symmetric 6×6):
[[6, 2, 0, 0, 0, 0],
 [2, 5, 1, 0, 0, 0],
 [0, 1, 4, 1, 0, 0],
 [0, 0, 1, 3, 1, 0],
 [0, 0, 0, 1, 2, 1],
 [0, 0, 0, 0, 1, 1]]

Expected Result (Approximate):
Dominant Eigenvalue λ ≈ 7.8013
Eigenvector v ≈ [0.56, 0.49, 0.36, 0.25, 0.13, 0.07]
Iterations ≈ 8, Converged = TRUE